BIOCARE MEDICAL

Material Safety Data Sheet






       Rev 091109
Page 1 of 3

1.
PRODUCT INFORMATION

Product:


Tween 20


Catalog #:


TW20, TWC20, TWN20, TWA20

Included in the following kits:
N/A

2.  
COMPOSITION / INFORMATION ON INGREDIENTS

	Component

	CAS No.

	Polyoxyethylenesorbitan Monolaurate
	9005-64-5

	-2-methyl-4-isothiazol-3-one
	2682-20-4

	5-chloro-2-methyl-4-isothiazol-3-one
	26172-55-4



Remainder of composition consists of non-hazardous ingredients.

3.  
HAZARDS IDENTIFICATION

Irritant to eyes and respiratory system.  Harmful if ingested.  Skin sensitizer, may cause skin irritation.
4.  
FIRST AID MEASURES


Seek medical attention immediately.

Eye Contact:  Flush eyes with copious amounts of water for at least 15 minutes.  Assure adequate flushing by separating eyelids with fingers.

Skin Contact:
  Remove contaminated clothing and shoes.  Flush skin with copious amounts of water for at least 15 minutes.  Wash contaminated clothing before reuse.

Ingestion:  Wash out mouth with water provided person is conscious.  Do not give anything by mouth if victim is unconscious.  Contact local poison control center.


Inhalation:  Remove victim to fresh air.  If not breathing, give artificial respiration.  If breathing is difficult, give oxygen.

5.  
FIREFIGHTING MEASURES

Extinguishing Media:  Extinguish using suitable agents appropriate for surrounding areas as per user’s firefighting prevention plan.

Fire and Explosion Hazards:  N/A

Firefighting Procedures:  Wear self-contained breathing apparatus and protective clothing.  Move container from fire area if possible.  Do not scatter spilled material with more water than needed for fire control.  Avoid breathing hazardous vapors.

6.  
ACCIDENTAL RELEASE MEASURES

Wear personal protective equipment, such as rubber gloves and boots, respirators, and goggles to prevent contact with self and others.  Use paper towels or absorbent materials to absorb spills.

7.  
HANDLING AND STORAGE


Handling:


See Section 8.


Storage:


See Instructions For Use.

8.  
EXPOSURE CONTROLS / PERSONAL PROTECTION


Eye Protection:

Wear goggles or safety glasses.  Use eyewash if necessary.


Skin Protection:

Wear latex or vinyl gloves.  Wash hands thoroughly after handling.


Respiratory Protection:
Wear respirator and avoid inhalation.


Ventilation:


Use in well-ventilated laboratory.


Other Protective Equipment:
Use lab coat to prevent contact with skin and clothing.

9.  
PHYSICAL AND CHEMICAL PROPERTIES


Appearance:


Yellow tint


Odor:


Characteristic soapy odor


pH:



N/A


Boiling point:


N/A


Flash point:


N/A

10.
STABILITY AND REACTIVITY


Stability:


See Instructions For Use.

Conditions/Materials to avoid:
Avoid elevated temperatures, sparks, open flame, all ignition sources, and incompatible materials.


Hazardous decomposition:
Tween 20 may form carbon dioxide and carbon monoxide when heated to decomposition.

11.
TOXICOLOGICAL INFORMATION


To the best of our knowledge, the chemical, physical, and toxicological properties of this material have not been thoroughly investigated.

12.
ECOLOGICAL INFORMATION


No information available at this time.

13.
DISPOSAL CONSIDERATIONS


Dispose in accordance with all applicable federal, state, and local environmental regulations.

14.
TRANSPORT INFORMATION


Please see product label to ensure proper transportation parameters such as temperature.

15.
REGULATORY INFORMATION


No information available at this time.

16.
OTHER INFORMATION

No products are to be construed as a recommendation for use in violation of any patents.  We make no representations, warranties, or assurances as to the accuracy or completeness of information provided on our data sheets and website.  Our warranty is limited to the actual price paid for the product.  Biocare Medical is not liable for any property damage, personal injury, time or effort, or economic loss caused by our products.

N/A = Not Available

[image: image1.png]


[image: image2.png]


[image: image3.png]o | ner




[image: image4.png]IVD



[image: image5.png]


     

             [image: image1.png]                                                                                                    

_1171785262

